import gdsfactory as gf

def make_waveguide(length=10):

c = gf.components.straight(length=length)

return c

waveguide = make_waveguide(length=25)

waveguide.show()

def make_bent_waveguide(length1=10,length2=20):
    c = gf.Component()
    wg1 = c.add_ref(gf.components.straight(length=length1))
    bend = c.add_ref(gf.components.bend_euler())
    wg2 = c.add_ref(gf.components.straight(length=length2))

    bend.connect("o1",wg1.ports["o2"])
    wg2.connect("o1", bend.ports["o2"])

    c.add_port("o1", port=wg1.ports["o1"])
    c.add_port("o2", port=wg2.ports["o2"])
    return c

| 1 | Python/GDSFactory environment |
| --- | --- |
| 2 | Generic PDK activation |
| 3 | gf.Component() |
| 4 | Standard GDSFactory components |
| 5 | add_ref() |
| 6 | Component vs reference |
| 7 | Optical ports |
| 8 | .connect() |
| 9 | .move() |
| 10 | Automatic routing |
| 11 | Cross-sections |
| 12 | Bend-radius constraints |
| 13 | Parent-level ports |
| 14 | Parameterized Python functions |
| 15 | @gf.cell |
| 16 | Cell caching |
| 17 | Hierarchical layout |
| 18 | Top-level components |
| 19 | Bounding boxes |
| 20 | GDS export |
| 21 | KLayout inspection |

import gdsfactory as gf

from gdsfactory.gpdk import get_generic_pdk

get_generic_pdk().activate()

import gdsfactory as gf

from gdsfactory.gpdk import get_generic_pdk

get_generic_pdk().activate()

@gf.cell

def make_bent_waveguide(length1=20, length2=30):

c = gf.Component()

wg1 = c.add_ref(

gf.components.straight(length=length1)

)

bend = c.add_ref(

gf.components.bend_euler()

)

wg2 = c.add_ref(

gf.components.straight(length=length2)

)

bend.connect("o1", wg1.ports["o2"])

wg2.connect("o1", bend.ports["o2"])

c.add_port("o1", port=wg1.ports["o1"])

c.add_port("o2", port=wg2.ports["o2"])

return c

top = gf.Component("top_chip")

device1 = make_bent_waveguide(20, 35)

device2 = make_bent_waveguide(40, 60)

ref1 = top.add_ref(device1)

ref2 = top.add_ref(device2)

ref2.move((100, 40))

top.add_port("A_in", port=ref1.ports["o1"])

top.add_port("A_out", port=ref1.ports["o2"])

top.add_port("B_in", port=ref2.ports["o1"])

top.add_port("B_out", port=ref2.ports["o2"])

print(list(top.ports))

print(top.bbox())

top.write_gds("chapter01_review.gds")